# SOP - Touch Calibration Setup on Torizon

Purpose
Run touch calibration from the Qt application while allowing the host to stop Docker containers, launch the calibration container, and reboot automatically.
do ssh torizon@192.168.225.ip
From step 1 to 5 do on host.

## 1. Create Host Calibration Script

Create /home/torizon/Precigo/touch_calibration.sh, 
add the calibration logic,  from this file "D:\SOP_Torizon\SOP_For_Touch_Calibration and update from application to host\touch_calibration.sh"
 then run:
chmod +x /home/torizon/Precigo/touch_calibration.sh

## 2. Create Host Monitor Script

Create /home/torizon/Precigo/calibration_monitor.sh 
add logic  from this : "D:\SOP_Torizon\SOP_For_Touch_Calibration and update from application to host\calibration_monitor.sh"
Run:
chmod +x /home/torizon/Precigo/calibration_monitor.sh

This used 
to monitor /tmp/start_touch_calibration and execute touch_calibration.sh.

## 3. Create systemd Service:

Use for pointing ExecStart to /home/torizon/Precigo/calibration_monitor.sh.

Create 
sudo nano /etc/systemd/system/calibration-monitor.service 
add logic from this file :
"D:\SOP_Torizon\SOP_For_Touch_Calibration and update from application to host\calibration-monitor.service"

## 4. Enable Service

Commands:
sudo systemctl daemon-reload
sudo systemctl enable calibration-monitor.service
sudo systemctl start calibration-monitor.service
Verify:
systemctl status calibration-monitor.service

## 5. Docker Compose

Mount /tmp into the application container:
- /tmp:/tmp

## 6. Qt Application

On button click create the trigger file:
open('/tmp/start_touch_calibration','w').close()

## 7. Runtime Flow

Button -> trigger file -> calibration monitor -> stop containers -> launch weston-touch-calibrator -> wait for completion -> stop calibration container -> reboot.

## 8. Verification

Check service logs:
journalctl -u calibration-monitor.service -f
Check containers:
docker ps

## 9. Troubleshooting

Permission denied: ensure scripts are executable.
Service not running: check systemctl status.
No reboot: verify reboot path using 'which reboot'.