- Add Loading.png and Weston.ini file on /home/torizon location.

- Nano Weston.ini add below content same as it is:

[core]

idle-time=0

[shell]

background-image=/home/torizon/Loading.png

background-type=scale

- Pass path to .yaml file to image torizon-weston same as below:

-/home/torizon/weston.ini:/etc/xdg/weston/weston.ini

- /home/torizon/Loading.png:/home/torizon/Loading.png

services:

weston:

image: torizon/arm32v7-debian-weston

restart: always

privileged: true

network_mode: host

volumes:

- /tmp:/tmp

- /dev:/dev

- /run/udev:/run/udev

-/home/torizon/weston.ini:/etc/xdg/weston/weston.ini

- /home/torizon/Loading.png:/home/torizon/Loading.png

cap_add:

- CAP_SYS_TTY_CONFIG

device_cgroup_rules:

- 'c 4:* rmw'

- 'c 13:* rmw'

- 'c 226:* rmw'

- 'c 10:223 rmw'

app:

image: intersense123/octo:3

privileged: true

restart: always

depends_on:

- weston

environment:

- QT_QPA_PLATFORM=wayland

- WAYLAND_DISPLAY=wayland-0

- XDG_RUNTIME_DIR=/tmp/1000-runtime-dir

volumes:

- /tmp:/tmp

- /dev:/dev

- /run/udev:/run/udev

- /media:/media

- /sys:/sys

- /etc/machine-id:/etc/machine-id:ro

- /etc/localtime:/etc/localtime:ro

- /etc/timezone:/etc/timezone:ro

- /var/run/systemd:/var/run/systemd

- /run/dbus:/run/dbus

- /home/torizon/Precigo/precigo.db:/home/torizon/app/precigo.db

- /home/torizon/Precigo/OctoPreciGo.db:/home/torizon/app/OctoPreciGo.db

- /home/torizon/Precigo/Octo_CSV:/home/torizon/app/Octo_CSV

devices:

- /dev/ttymxc0

- /dev/rtc0

- /dev/rtc1

cap_add:

- SYS_TIME

- NET_ADMIN

- SYS_BOOT

device_cgroup_rules:

- 'c 13:* rmw'

network_mode: host

# To Get remote access do Below steps:

- Create folder custom -> add Loading.png and weston.ini in it.

- On weston.ini :

[core]

idle-time=0

modules=screen-share.so

[shell]

background-image=/home/torizon/custom/Loading.png

background-type=scale

3)on docker-compose.yml
services:

weston:

image: torizon/weston:$CT_TAG_WESTON

restart: always

privileged: true

network_mode: host

ipc : host

environment:

- ENABLE_RDP=1

- RDP_PORT=3389

command: >

--developer

--tty=/dev/tty7

--

--config=/etc/xdg/weston/weston.ini

--use-pixman

volumes:

- /tmp:/tmp

- /dev:/dev

- /run/udev:/run/udev

- /home/torizon/custom:/etc/xdg/weston/

- /home/torizon/custom/Loading.png:/home/torizon/custom/Loading.png

cap_add:

- CAP_SYS_TTY_CONFIG

device_cgroup_rules:

- 'c 4:* rmw'

- 'c 13:* rmw'

- 'c 226:* rmw'

- 'c 10:223 rmw'

After that click on search on Pc and open remote Desktop Connection always use port :3389 -> click connect